# 3. Methodology

## 3.1 Dataset

This project uses the WESAD (Wearable Stress and Affect Detection) dataset [1], comprising physiological signals from 15 subjects (S2-S17, excluding S1 and S12 per the original dataset release) recorded via chest-worn (RespiBAN, 700Hz: ECG, EDA, EMG, Temperature, Respiration, three-axis acceleration) and wrist-worn (Empatica E4) devices during a laboratory protocol inducing Baseline, Stress, and Amusement affective states. This project uses the chest-worn signal set exclusively, consistent with the proposal's scope. Three affective classes were retained (Baseline, Stress, Amusement), with transient and meditation-condition labels dropped, following Schmidt et al.'s [1] labelling protocol.

## 3.2 Preprocessing and Feature Extraction

Each chest signal channel was bandpass-filtered using a third-order Butterworth filter with channel-appropriate cutoff frequencies (ECG: 0.5-40Hz; EMG: 20-250Hz; Respiration: 0.1-0.35Hz), applied via zero-phase filtering (filtfilt) to avoid phase distortion. Signals were segmented into 60-second windows with 50% overlap, with each window assigned the majority label across its span; windows without a majority Baseline, Stress, or Amusement label were discarded. For Random Forest and XGBoost, each windowed channel was reduced to nine statistical, time-domain, and frequency-domain features (mean, standard deviation, min, max, skewness, kurtosis, zero-crossing rate, peak frequency, and mean power spectral density via Welch's method), yielding a 45-dimensional feature vector (5 channels x 9 features) per window. For the LSTM, raw normalised signal sequences were retained per window and downsampled by a factor of 100 (from 42,000 to 420 timesteps per window) for computational tractability. This pipeline produced 1,105 total windows across all 15 subjects (587 Baseline, 332 Stress, 186 Amusement).

## 3.3 Preprocessing Correction

During pipeline validation, systematic NaN values were identified in all Respiration-derived statistical features. Root-cause analysis attributed this to numerical instability in the Butterworth filter's transfer-function ('ba') coefficient representation for the narrow Respiration passband (0.1-0.35Hz) relative to the 700Hz sampling rate's Nyquist frequency. The filter was reformulated using second-order sections (SOS), which is numerically stable for this configuration, and the full pipeline was re-run. This correction is detailed further, with its measured performance impact, in Section 4.0.

## 3.4 Model Architectures and Training

Three candidate models were implemented: Random Forest and XGBoost (scikit-learn and xgboost libraries respectively) operating on the 45-dimensional engineered feature vectors, and a two-layer LSTM (Keras/TensorFlow) operating on raw downsampled sequences, with dropout (0.3) between layers and class-weighted loss (computed via sklearn's balanced class-weighting) to address the moderate class imbalance (53% Baseline, 30% Stress, 17% Amusement). Table 6 details the hyperparameters used for each model.

| Model | Hyperparameters used | Basis |
| --- | --- | --- |
| Random Forest | n_estimators=300, max_depth=15, random_state=42 | Common defaults for tabular physiological feature sets in prior literature; not tuned via grid search |
| XGBoost | n_estimators=300, max_depth=6, learning_rate=0.1 | XGBoost library defaults with minor adjustment; not tuned via grid search |
| LSTM | 2-layer LSTM, 32 units/layer, dropout 0.3, Adam optimiser, early stopping (patience=5), class-weighted loss | Simple architecture matching proposal's 1-2 layer, 64-128 unit search range; not tuned via Keras Tuner |

Table 6: Hyperparameters used for each model.

The project proposal specified formal hyperparameter optimisation via grid search (Random Forest, XGBoost) and Keras Tuner (LSTM). Due to project timeline constraints, this formal search was not conducted; instead, literature-informed fixed hyperparameters were used across all three models. This is disclosed as a limitation in Section 6 and represents a legitimate simplification anticipated by the project's own risk register, which noted that nested hyperparameter search within LOSO cross-validation is computationally expensive and that a single fixed-parameter run was an acceptable fallback if time-constrained.

## 3.5 Evaluation Protocol

All models were evaluated using Leave-One-Subject-Out (LOSO) cross-validation: for each of the 15 subjects, a model was trained on the remaining 14 subjects and tested on the held-out subject, repeated for all 15 subjects. This protocol was chosen specifically to measure cross-subject generalisation, which is the primary practical requirement for a deployable stress-detection system that must work on users unseen during training, and is consistent with Siirtola and Roning's [4] finding that person-independent evaluation reveals substantially larger performance drops than person-dependent evaluation. Accuracy, precision, recall, and macro-averaged F1-score were computed per fold and averaged across all 15 folds. Statistical significance of pairwise model differences was assessed using the Wilcoxon signed-rank test on matched per-subject scores, with matched-pairs rank-biserial correlation computed as an effect size measure. Full results are reported in Section 4.

## 3.6 Deployment Methodology

The best-performing model by the combined criteria of predictive accuracy and inference latency was serialised and deployed behind a Flask REST API, with a React-based frontend dashboard consuming that API to demonstrate real-time prediction on sample sensor windows. Full implementation and testing details are reported in Section 5.